- Explain Switch

ANS - A switch is a networking device that connects multiple devices (PCs, printers, servers) within a LAN (Local Area Network). It operates at Layer 2 (Data Link Layer) of the OSI model and uses MAC addresses to forward frames to the correct destination.

- Key Functions of a Switch

- Learning: Reads source MAC addresses and stores them in the MAC address table.

-Forwarding/Filtering: Sends frames only to the intended destination port rather than broadcasting to all ports.

- Loop Prevention: Uses Spanning Tree Protocol (STP).

- Segmentation: Each port is its own collision domain.

- Explain Switch Boot Sequence?

ANS - The boot process of a Cisco switch occurs in a defined order:

Switch Boot Sequence

-POST (Power-On Self-Test):

Tests hardware (CPU, memory, ports).

- Load the Boot Loader:

A small program stored in ROM that allows the switch to find and load the IOS.

-Locate and Load Cisco IOS:

The switch looks for the IOS in flash memory and loads it.

- Load the Startup Configuration:

- The switch loads configuration from NVRAM (startup-config).

- If no startup-config exists, it enters initial configuration dialog (setup mode).

3. Explain Three Methods to Access Switch Command Line Interface ?

ANS - Cisco switches allow access to the CLI (Command Line Interface) using:

1. Console Access (Out-of-band)

- Requires a console cable connected directly to the switch.

- Used for initial configuration or password recovery.

- No network required.

2. Telnet Access (In-band)

- Remote text-based access over the network using Telnet protocol.

- Requires IP addressing and VTY line configuration.

- Not encrypted → insecure.

3. SSH Access (In-band)

- Secure, encrypted remote access.

- Replaces Telnet for secure management.

- Requires hostname, domain name, and RSA key configuration.

4. Explain and Configuring the Cisco Internet Operating System (Cisco IOS)?

- What is Cisco IOS?

- Cisco IOS is the operating system that runs on Cisco routers and switches. It provides:

3 CLI interface

- Network services (routing, switching, security)

- Device management features

- Basic Cisco IOS Configuration

- Below is a common set of initial switch configurations:

1. Set hostname

- Switch(config)# hostname S1

2. Secure privileged EXEC mode

- S1(config)# enable secret cisco123

3. Set console password

- S1(config)# line console 0

- S1(config-line)# password cisco

- S1(config-line)# login

4. Configure VTY (Telnet/SSH) password

- S1(config)# line vty 0 15

- S1(config-line)# password cisco

- S1(config-line)# login

5. Configure SSH

- S1(config)# hostname S1

- S1(config)# ip domain-name example.com

- S1(config)# crypto key generate rsa

- S1(config)# ip ssh version 2

- S1(config)# line vty 0 15

- S1(config-line)# transport input ssh

6. Configure management VLAN interface (SVI)

- S1(config)# interface vlan 1

- S1(config-if)# ip address 192.168.1.2 255.255.255.0

- S1(config-if)# no shutdown

7. Save configuration

- S1# copy running-config startup-config

5. Explain Switch Port?

ANS - A switch port is a physical interface on a switch used to connect end devices or other networking devices.

- Types of Switch Ports

- Access Port

- Belongs to one VLAN

Connects to end devices (PCs, printers)

- Switch(config)# interface f0/1

- Switch(config-if)# switchport mode access

- Switch(config-if)# switchport access vlan 10

Trunk Port

Carries multiple VLANs using tagging (802.1Q- Connects switches/routers

- Switch(config)# interface f0/24

- Switch(config-if)# switchport mode trunk

- Switch(config-if)# switchport trunk allowed vlan 10,20,30

Dynamic Ports (Dynamic Auto / Desirable)

- Negotiate trunking using DTP

- Not commonly used in modern networks.

- Port States (STP)

- Blocking

- Listening

- Forwarding

- Disable

3-enable secret [password] is

hashed using the algorithm.

A. MD5

B. AH

C. PSK

D. ESP

E. WPA2

ANS-  MD5

4- An engineer connects to Router R1 and issues a show ip ospf neighbor command.

The status of neighbor 2.2.2.2 lists FULL/BDR. What does the BDR mean?

A. R1 is an Area Border Router.

B. R1 is a backup designated router.

C. Router 2.2.2.2 is an Area Border Router.

D. Router 2.2.2.2 is a backup designated router.

ANS – Router 2.2.2.2 is a backup designated

5-Which command is used to view the neighbor discovery table on a PC?

A. show ipv6 neighbor

B. show ipv6 neighbors

C. netsh interface ipv6 show neighbor

D. netsh interface ipv6 show neighbors

ANS -  netsh interface ipv6 show neighbor

6-What type of variable is being shown? Routers = [R1,R2,R3]

A. List

B. Dictionary

C. Simple

D. Unsigned integers

ANS - LIST

7- Identify the fields in an IPv4 header. (Choose three)

A. Host component

B. Time to Live

C. Source address

D. Destination address

E. Network address

ANS

- TIME TO LIVE

- SOURCE ADDRESS

- DESTINATION ADDRESS